NEVARA MRV Platform

# NEVARA Ecological Intelligence Report

Tahir · River Restoration · 2026

Confidential Ecological Monitoring Report  ·  Generated 12 June 2026  ·  NEVARA MRV v2.0

# Executive Summary

Ecological assessment for Tahir, Karnataka, India, based on Landsat 8 Collection 2 SR imagery for the 2026-04-27 to 2026-06-11 window. Current condition: The composite ecosystem health index of 43/100 reflects a fair overall ecological condition. Note: some indices used fallback imagery this cycle, which slightly reduces confidence. The composite ecosystem health index stands at 43 out of 100, with NDVI down 65.2% versus baseline. Key improvements: no statistically notable positive shifts were detected this cycle relative to the available baseline. Key risks: mean NDVI of 0.191 indicates sparse vegetation. The area shows limited live canopy relative to a healthy ecosystem baseline. vegetation indicators have declined relative to earlier cycles over 7 monitored years (2020–2026). The NDVI trajectory is stable, which warrants closer attention. water availability indicators show signs of stress in the monitored area. Mean canopy moisture (NDMI) of -0.193 is below the stress threshold of −0.15. ecological risk is rated 58/100 (elevated urgency). The site is stable but shows risk factors that warrant attention. risk indicators recommend a increased monitoring cadence, above the current quarterly schedule. Recommended attention areas: Investigate causes of low canopy density and consider targeted planting or protective interventions. Prioritise a field check to identify drivers of vegetation loss (drought, clearing, salinity, or grazing pressure). Assess hydrological inputs and consider whether supplementary watering or tidal/flow restoration is needed. Address limiting factors (moisture, soil, slope) before committing major restoration resources. Monitoring outlook: Risk indicators recommend a increased monitoring cadence, above the current quarterly schedule. The next monitoring cycle is scheduled for 9/9/2026. All statements above are derived solely from the evidence metrics for this monitoring run and carry traceable source references.

## Ecological Intelligence KPI Summary

| Metric | Value |
| --- | --- |
| Ecosystem Health Index | 42.9 / 100 |
| Vegetation Density (NDVI) | 0.1912 |
| Ecological Risk Score | 57.66 / 100 |
| Restoration Suitability Score | 30.08 / 100 |
| Vegetation Trend | stable |
| Annual NDVI Rate of Change | 0.0004 NDVI/yr |
| Intervention Urgency | ELEVATED |
| Monitoring Cycles Completed | 7 |
| Evidence Validation Status | WARNING (97 / 100) |

# 1. Project Overview

This monitoring cycle was conducted using Landsat 8 Collection 2 SR imagery. Analysis type: scheduled. A cloud-cover filter of 30% was applied to ensure data quality. All spectral indices and derived metrics were calculated from atmospherically corrected surface reflectance data.

## Project & Monitoring Details

| Metric | Value |
| --- | --- |
| Project Name | Tahir |
| Organisation | Alice Johnson |
| Location | Karnataka, India |
| Ecosystem Type | River Restoration |
| Site Area | 4.4346 ha |
| Monitoring Cycle Type | scheduled / edaa5fa5-3a8e-4428-adf8-8db70f559a61 |
| Primary Satellite Dataset | Landsat 8 Collection 2 SR |
| Analysis Period | 2026-04-27 → 2026-06-11 |
| Cloud Cover Filter Applied | 30% |
| Report Generated | 12 June 2026 |

# 2. Project Boundary & Site Overview

## Boundary & Spatial Metrics

| Metric | Value |
| --- | --- |
| Site Area | 4.4346 ha |
| Site Area (sq m) | 44346 m² |
| Perimeter | 0.9704 km |
| Centroid Latitude | 16.1453° |
| Centroid Longitude | 75.929° |

Figure: Figure 1 — Project Boundary: GIS-delineated site extent with terrain context evidence asset

Figure: Figure 2 — True Colour Composite: Sentinel-2 SR (B4/B3/B2) visual canopy and land-cover verification evidence asset

# 3. Vegetation Health Analysis

Evidence Summary: Mean NDVI = 0.191; Mean EVI = 0.096; NDVI trend direction = stable; Trend strength (NDVI/yr) = 0.0004; NDVI change vs baseline = -65.2 %; Mean NDMI (canopy moisture) = -0.193; Mean NDWI (surface water) = -0.332. Interpretation: Mean NDVI of 0.191 indicates sparse vegetation. The area shows limited live canopy relative to a healthy ecosystem baseline. Vegetation indicators have declined relative to earlier cycles over 7 monitored years (2020–2026). The NDVI trajectory is stable, which warrants closer attention. Water availability indicators show signs of stress in the monitored area. Mean canopy moisture (NDMI) of -0.193 is below the stress threshold of −0.15. Recommendation: Investigate causes of low canopy density and consider targeted planting or protective interventions. Prioritise a field check to identify drivers of vegetation loss (drought, clearing, salinity, or grazing pressure). Assess hydrological inputs and consider whether supplementary watering or tidal/flow restoration is needed.

## Multi-Index Vegetation Health Assessment

| Metric | Value |
| --- | --- |
| NDVI Mean (Vegetation Density) | 0.1912 |
| NDVI Minimum | 0.1312 |
| NDVI Maximum | 0.2512 |
| EVI Mean (Enhanced Vegetation) | 0.0965 |
| SAVI Mean (Soil-Adjusted Vegetation) | 0.1053 |
| NDMI Mean (Moisture Index) | -0.1927 |
| NBR Mean (Burn Ratio) | -0.1071 |
| BSI Mean (Bare Soil Index) | 0.2342 |

Figure: Figure 3 — NDVI Vegetation Density Map: Sentinel-2 SR (B8/B4) · Dark green = dense canopy (NDVI > 0.6), red = bare or stressed cover (NDVI < 0.1) evidence asset

# 4. Water Content & Moisture Analysis

Water content and moisture conditions across the project site were assessed using the Normalised Difference Water Index (NDWI) and Normalised Difference Moisture Index (NDMI) derived from Sentinel-2 surface reflectance imagery. NDWI of -0.332 indicates dry or water-stressed conditions across the majority of the site. NDMI of -0.193 characterises foliar water content and drought stress in the vegetation canopy. Open water and wetland areas comprise 0.0% of the classified land cover. Hydrological stress score: 12/100.

## Water & Moisture Index Summary

| Metric | Value |
| --- | --- |
| NDWI Mean (Water Index) | -0.3321 |
| NDMI Mean (Moisture Index) | -0.1927 |
| Water Body Coverage | 0% |
| Hydrological Stress Score | 12.25 / 100 |
| Water Retention Score | 87.75 / 100 |

Figure: Figure 4 — NDWI Moisture Map: Sentinel-2 SR (B3/B8) · Blue = high moisture / open water, brown = dry stress evidence asset

# 5. Land Surface Temperature Analysis

## Land Surface Temperature (LST) — Landsat 8/9 Thermal Infrared

| Metric | Value |
| --- | --- |
| LST Mean | N/A |
| LST Minimum | 48.58 °C |
| LST Maximum | 52.52 °C |
| Thermal Range | 3.94 °C |
| Source Dataset | LANDSAT/LC08+LC09/C02/T1_L2 |

Figure: Figure 5 — Land Surface Temperature Map: Landsat 8/9 Collection 2 (ST_B10) · Blue = cool (< 25 °C), orange-red = thermal stress (> 40 °C) evidence asset

# 6. Land Use & Cover Classification

Land use and land cover composition was classified using the Google Dynamic World v1 dataset. Vegetation cover comprises 100.0% of the analysis area. Open water and wetland areas account for 0.0%. Built-up and impervious surfaces occupy 0.0%. Bare and sparsely vegetated ground covers 0.0%.

## Land Use / Land Cover Composition — Dynamic World v1

| Metric | Value |
| --- | --- |
| Vegetation Cover | 100% |
| Open Water & Wetland | 0% |
| Barren / Sparse Vegetation | 0% |
| Built-up / Impervious | 0% |
| Other Land Cover | 0% |
| Classification Dataset | GOOGLE/DYNAMICWORLD/V1 |

Figure: Figure 6 — Land Cover Classification: Google Dynamic World v1 · Green = vegetation, blue = water, grey = built-up, tan = bare ground evidence asset

# 7. Multi-Year NDVI Trend Analysis

Evidence Summary: NDVI trend direction = stable; Trend strength (NDVI/yr) = 0.0004; NDVI change vs baseline = -65.2 %. Interpretation: Vegetation indicators have declined relative to earlier cycles over 7 monitored years (2020–2026). The NDVI trajectory is stable, which warrants closer attention. Recommendation: Prioritise a field check to identify drivers of vegetation loss (drought, clearing, salinity, or grazing pressure).

## Historical Vegetation Trend Summary

| Metric | Value |
| --- | --- |
| Baseline Year | 2020 |
| Current Year | 2026 |
| Trend Direction | STABLE |
| Annual NDVI Rate of Change | 0.0004 NDVI / year |
| Years in Analysis Window | 7 |
| Dataset | Sentinel-2 SR Harmonized (COPERNICUS/S2_SR_HARMONIZED) |

## Annual NDVI Trajectory

| Metric | Value |
| --- | --- |
| Year | NDVI Mean | Year-on-Year Change |
| 2020 | 0.2092 | Baseline year |
| 2021 | 0.2127 | 0.0035 |
| 2022 | 0.2402 | 0.0275 |
| 2023 | 0.231 | -0.0092 |
| 2024 | 0.2513 | 0.0203 |
| 2025 | 0.2251 | -0.0262 |
| 2026 | 0.1939 | -0.0312 |

Figure: Figure 7 — Annual NDVI Trend: Sentinel-2 SR median composites with vegetation health threshold bands (dense > 0.6, moderate > 0.4, sparse > 0.2) evidence asset

# 8. Terrain Hydrology & Drainage Assessment

Evidence Summary: Runoff risk = low; Hydrological stress score = 12 /100; Mean slope = n/a. Interpretation: Runoff risk is rated low (stress score 12/100), indicating stable hydrological conditions. Recommendation: No specific action is required for this indicator in the current cycle.

## Terrain Hydrology — SRTM 30m Digital Elevation Model

| Metric | Value |
| --- | --- |
| Mean Elevation | N/A |
| Minimum Elevation | 524 m |
| Maximum Elevation | 529 m |
| Mean Terrain Slope | N/A |
| Slope Class | GENTLE |
| Surface Runoff Risk | LOW |
| Water Retention Score | 87.75 / 100 |
| Hydrological Stress Score | 12.25 / 100 |

Figure: Figure 8 — Terrain Hydrology Map: SRTM 30m DEM · Blue = drainage channels / low runoff zones, green-yellow = moderate terrain, red = high hydrological stress evidence asset

# 9. Ecological Restoration Suitability

Evidence Summary: Restoration suitability = 30 /100; Restoration priority = low. Interpretation: Restoration suitability scores 30/100 (low priority). Site conditions currently constrain restoration potential. Recommendation: Address limiting factors (moisture, soil, slope) before committing major restoration resources.

## Restoration Suitability Assessment — Multi-Index Composite

| Metric | Value |
| --- | --- |
| Overall Suitability Score | 30.08 / 100 |
| Priority Classification | LOW |
| Vegetation Condition Score | 34.37 / 100 |
| Moisture Condition Score | 12.74 / 100 |
| Soil Condition Score | 29.46 / 100 |
| Terrain Slope Condition | 50 / 100 |
| Methodology Version | restoration-suitability-v2.0 |

Figure: Figure 9 — Restoration Suitability Map: Multi-index composite (NDVI + NDMI + BSI + terrain slope) · Dark green = high suitability (> 70), red = low suitability (< 40) evidence asset

# 10. Ecological Risk Assessment

Evidence Summary: Overall ecological risk = 58 /100; Intervention urgency = elevated; Monitoring priority = increased. Interpretation: Ecological risk is rated 58/100 (elevated urgency). The site is stable but shows risk factors that warrant attention. Recommendation: Maintain standard monitoring and track whether risk drivers trend upward in the next cycle.

## Ecological Risk — Weighted Multi-Domain Assessment

| Metric | Value |
| --- | --- |
| Overall Ecological Risk Score | 57.66 / 100 |
| Vegetation Risk Component | 76.4 / 100 |
| Moisture Risk Component | 94.15 / 100 |
| Thermal Risk Component | 50 / 100 |
| Hydrological Risk Component | 12.25 / 100 |
| Historical Trend Risk | 45 / 100 |
| Restoration Priority | MODERATE |
| Intervention Urgency | ELEVATED |
| Monitoring Priority | INCREASED |
| Confidence Score | 95 / 100 |
| Methodology Version | ecological-risk-v1.0 |

Figure: Figure 10 — Ecological Risk Map: Aggregated multi-domain risk indicators · Green = low risk (< 40), amber = moderate (40–70), red = high risk (> 70) evidence asset

# 11. Key Ecological Findings

The following structured observations were generated by the NEVARA deterministic rules engine from evidence metrics. Each finding is directly grounded in satellite-derived indicator values.

## vegetation

| Severity | Metric | Value | Finding | Recommendation | Confidence |
| --- | --- | --- | --- | --- | --- |
| high | vegetation.ndvi_mean | 0.1912 | NDVI_BELOW_VEGETATION_HEALTH_THRESHOLD | PRIORITIZE_FIELD_VERIFICATION_AND_REVEGETATION_PLANNING | 90 |
| high | vegetation.ndmi_mean | -0.1927 | NDMI_INDICATES_VEGETATION_MOISTURE_STRESS | REVIEW_WATER_RETENTION_AND_SOIL_MOISTURE_CONDITIONS | 86 |
| moderate | vegetation.bsi_mean | 0.2342 | BSI_INDICATES_ELEVATED_BARE_SOIL_CONDITION | TARGET_SOIL_STABILIZATION_AND_GROUND_COVER_REVIEW | 84 |

## land use

| Severity | Metric | Value | Finding | Recommendation | Confidence |
| --- | --- | --- | --- | --- | --- |
| low | lulc.vegetation_pct | 100 | LULC_INDICATES_VEGETATION_COVER_PRESENT | MAINTAIN_LAND_COVER_MONITORING | 80 |

## historical change

| Severity | Metric | Value | Finding | Recommendation | Confidence |
| --- | --- | --- | --- | --- | --- |
| moderate | historical.trend_direction | stable | HISTORICAL_NDVI_TREND_DOES_NOT_INDICATE_DECLINE | MAINTAIN_TREND_MONITORING | 84 |

## hydrology

| Severity | Metric | Value | Finding | Recommendation | Confidence |
| --- | --- | --- | --- | --- | --- |
| low | hydrology.runoff_risk | low | HYDROLOGY_RUNOFF_RISK_NOT_HIGH | CONTINUE_HYDROLOGY_MONITORING | 82 |
| low | hydrology.hydro_stress_score | 12.25 | HYDROLOGICAL_STRESS_SCORE_REQUIRES_ROUTINE_TRACKING | TRACK_HYDROLOGICAL_STRESS_IN_NEXT_MONITORING_CYCLE | 82 |

## restoration

| Severity | Metric | Value | Finding | Recommendation | Confidence |
| --- | --- | --- | --- | --- | --- |
| high | restoration.suitability_score | 30.08 | RESTORATION_SUITABILITY_SCORE_IS_LOW | REVIEW_CONSTRAINTS_BEFORE_ACTIVE_RESTORATION | 80 |

## risk

| Severity | Metric | Value | Finding | Recommendation | Confidence |
| --- | --- | --- | --- | --- | --- |
| moderate | risk.overall_risk_score | 57.66 | OVERALL_ECOLOGICAL_RISK_IS_NOT_HIGH | FOLLOW_ASSIGNED_MONITORING_PRIORITY | 95 |

# 12. Recommendations & Actions

Evidence-based recommendations derived from this monitoring cycle. Vegetation cover is sparse: Investigate causes of low canopy density and consider targeted planting or protective interventions. Vegetation health is declining: Prioritise a field check to identify drivers of vegetation loss (drought, clearing, salinity, or grazing pressure). Water availability shows signs of stress: Assess hydrological inputs and consider whether supplementary watering or tidal/flow restoration is needed. Limited restoration suitability: Address limiting factors (moisture, soil, slope) before committing major restoration resources. Ecological risk is moderate: Maintain standard monitoring and track whether risk drivers trend upward in the next cycle. Increased monitoring frequency recommended: Increase the monitoring frequency for this project until risk indicators stabilise.

## Priority Action Register — Critical & High Severity Findings

| Metric | Value |
| --- | --- |
| Priority | Ecological Finding | Recommended Action | Urgency |
| 1 | NDVI_BELOW_VEGETATION_HEALTH_THRESHOLD | PRIORITIZE_FIELD_VERIFICATION_AND_REVEGETATION_PLANNING | HIGH |
| 2 | NDMI_INDICATES_VEGETATION_MOISTURE_STRESS | REVIEW_WATER_RETENTION_AND_SOIL_MOISTURE_CONDITIONS | HIGH |
| 3 | RESTORATION_SUITABILITY_SCORE_IS_LOW | REVIEW_CONSTRAINTS_BEFORE_ACTIVE_RESTORATION | HIGH |

# 13. Evidence Validation & Data Quality

All evidence assets in this report were validated through the NEVARA evidence validation layer prior to report generation. The following summary confirms data completeness and integrity for this monitoring cycle.

## Evidence Package Validation Summary

| Metric | Value |
| --- | --- |
| Status | warning |
| Score | 97 |
| Validated At | 2026-06-12T08:02:58.423Z |
| Warnings | insights.json exists on disk but is not listed in manifest assets |
| Errors | None |
| Missing Assets | None |
| Generated Assets | boundary-overview.png, evidence-manifest.json, historical-trend.png, hydrology.png, insights.json, lst.png, lulc.png, metrics.json, ndvi.png, ndwi.png, observations.json, restoration.png, risk.png, true-color.png |